PB-1000 Pro QUICK SPECS:

- Driver: 12”

- Amplifier: 325 watts RMS (820 watts peak), Class D

- Frequency Response:

  - 17Hz to 260Hz Standard +/-3dB

  - 19Hz to 260Hz Sealed +/-3dB

  - In Room Low Freq – 18Hz

- Dimensions:

  - Weight – 42.5lbs (19.3kg)

  - With Grille – 18.9” (479mm) H X 15.0” (380mm) W X 20.0” (509mm) D

  - Without Grille – 18.9” (467mm) H X 15.0” (380mm) W X 19.2” (488mm) D

FULL SPECIFICATIONS:

- Subwoofer Specs

  - Protective non-resonant extra-thick ABS fabric mesh grill

  - Iso-elastomer feet included

  - Extra heavy-duty, high current power cord 6'

  - Quick Start Guide

  - Port Plugs (free upon request)

  - Unbalanced stereo input/output

  - Speaker Level Stereo Input

  - Auto/On and Trigger options through app, 3-12V AC/DC trigger

  - RoHS compliant, lead-free construction, world-wide safety certifications

  - Green 0.5-watt low standby power consumption

- Driver Specs

  - All-new proprietary 12-inch long-throw driver with high-precision tuned overhung motor configuration.

  - 9.26lb (4.2kg) pound Woofer with 6.91lb (3.14kg) pounds motor

  - 13.1mm Xmax, 26.8mm Xmech precision aligned excursion

  - 1.5” diameter Voice Coil with high current 4-layer high-purity copper wire windings

  - Premium high-purity copper voice coil winding design minimizes distortion and power compression with maximum heat dissipation, linearity, and tuned magnetics for high excursion

  - Two-piece high grade oversized Y35 Ferrite magnet motor

  - High Temperature 3003-H18 aluminum former/bobbing

  - 6” composite cotton-poly extreme excursion linear spider

  - Dual 24-strand copper-alloy high-temperature leads

  - Proprietary injection molded gasket and NBR extreme-excursion surround

  - Lightweight treated Fiber Composite cone for enhanced sensitivity and transient response

  - Top plate assembly with low carbon 1010 steel for optimal magnetics

  - Oversized low carbon 1008 steel T-yoke with integrated pole piece extension for maximum cooling and electromagnetics

  - 12” premium powder coated basket with FEA optimized analysis for maximum rigidity, tensile strength, and energy transference

  - Oversized motor system for driving full excursion and industry best SPL, with pinpoint transient speed

  - Precision machining for motor and narrow tolerance suspension parts ensure flawless linearity at full range of excursion

- Amplifier Specs

  - 325 watts RMS, 820+ watts peak power output Class D design

  - 50Mhz Analog Devices Audio DSP, high resolution double precision 56bit filtering

  - Wide input voltage bandwidth compliant for consumer and pro audio up to 4V line level input.

  - High efficiency LLC resonant SMPS with 17A 650V MOSFET, resonant switching results in lower EMI and highest efficiency.

  - Cool-running Class D amplifier switching topology with massive 34A 120V MOSFET’s

  - SVS BLE control app for iOS and Android

    - 3 complete User presets – name-able, customizable

    - iOS (iphone5 or newer) or Android (4.42 or newer)

    - 3 PEQ’s per preset

    - Adjustable low pass, phase, polarity, 3 PEQs, room gain compensation, port tuning control for best alignment with speakers

    - Standard and sealed port tuning modes

  - Customer EQ and DSP limiter settings specifically for the 1000 Pro

  - Input impedance 16kohm (unbalance RCA)

  - Input impedance 20kohm (speaker level)

  - New Intelligent Feature control (IFC) user interface on the amplifier with bright blue 11 LED bar display and easy push-button controls.